Operation Manual

Flux-tool cleaning system

PC Software

Production : ATK5 EE

S/W Engineer : Hyun-su Park

- Run a program

Double-click the FluxTool_CleanerSystem_ATV.exe icon on the desktop to run it

- Log-in

Log in after entering ID and password

- Default ID : 000000

- Default Password : 1

- Operation

1

  - Close the front door of the equipment and press the start button

If the following window opens, press the check button

2

  - Select the Recipe file and press the OK button

  - If you want to stop the process, press the stop button

- Ch1, Ch2, Ch3 The same GUI and SEQ

1

Maintenance

If you press the Maintenance button, you will see the screen above

The process can be carried out the same as the "Operation" screen

- Individual I/O operations are possible

However, it is impossible to operate during the process

3

2

1

Recipe

  - New / Edit / Copy / Save / Delete / Cancel button

- New : Create a new Recipe file

- Edit : Modifying Recipe Content

- Copy : Copy Recipe File

- Save : Create or modify Recipe and save it as a file (csv file)

- Delete : Delete Recipe File

- Cancel : Cancel creating or modifying Recipe

  - Add / Insert / Delete

- Ability to add, insert, or delete each step in Recipe

- One recipe can be created up to 50 steps in total

  - Recipe Item

- Water : Whether to use water

- Air knife : Whether to use dry air

- Pin move : Whether to use pin

- Up : Keep Pin up

- Down : Keep Pin down

- Up/Down : Repeat Pin up/down every 3 seconds

- Time : Each step process time (sec)

- Configure

Cylinder fwd/bwd time out : Alarm if it does not operate within the set time when moving the cylinder

- In/Output

Check I/O control and status

- Alarm

Alarm history by channel

- Alarm list

  - Code. 1002 : Failed to read recipe file

- Alarm when the Recipe file fails to be loaded during the process

  - Code. 1005 : Cylinder forward time out

- Alarm if cylinder operation fails to reach forward position within set time

  - Code. 1006 : Cylinder backward time out

- Alarm if cylinder operation fails to reach backward position within set time

  - Code. 1007 : Cylinder home time out

- Alarm if cylinder operation fails to reach home position within set time

- Event Log

Event history by channel (year/month/date)

- User regist

New User registration and modify, delete function

- User regist

- Employee number : Employee number or unique numbers

- Name : Name

- Authority : Setting permissions (screen access permission)

- Password : Password

- User edit

- Modify existing registered user information

- User delete

- Delete registered users